Facts About Mold

Why is mold growing in my home?

Molds are part of the natural environment. Outdoors, molds play a part in nature by breaking down dead organic matter such as fallen leaves and dead trees, but indoors, mold growth should be avoided. Molds reproduce by means of tiny spores; the spores are invisible to the naked eye and float through outdoor and indoor air. Mold may begin growing indoors when mold spores land on surfaces that are wet. There are many types of mold, and none of them will grow without water or moisture.

Can mold cause health problems?

Molds have the potential to cause health problems. Molds produce allergens (substances that can cause allergic reactions), irritants, and in some cases, potentially toxic substances (mycotoxins). Inhaling or touching mold or mold spores may cause allergic reactions in sensitive individuals. Allergic responses include hay fever--‐type symptoms, such as sneezing, runny nose, red eyes, and skin rash (dermatitis). Allergic reactions to mold are common. They can be immediate or delayed. Molds can also cause asthma attacks in people with asthma who are allergic to mold. In addition, mold exposure can irritate the eyes, skin, nose, throat, and lungs of both mold--‐allergic and non--‐allergic people. Research on mold and health effects is ongoing. For more detailed information consult a health professional.

How do I get rid of mold?

It is impossible to get rid of all mold and mold spores indoors; some mold spores will be found floating through the air and in house dust. The mold spores will not grow if moisture is not present. Indoor mold growth can and should be prevented or controlled by controlling moisture indoors. If there is mold growth in your home, you must clean up the mold and fix the water problem.

Mold Cleanup Guidelines

The tips and techniques presented in this section will help you clean up your mold problem. Professional cleaners or remediators may use methods not covered in this publication. Please note that mold may cause staining and cosmetic damage. It may not be possible to clean an item so that its original appearance is restored.

- Fix plumbing leaks and other water problems as soon as possible. Dry all items completely.

- Scrub mold off hard surfaces with detergent and water, and dry completely.

- Absorbent or porous materials, such as ceiling tiles and carpet, may have to be thrown away if they become moldy. Mold can grow on or fill in the empty spaces and crevices of porous materials, so the mold may be difficult or impossible to remove completely.

- Do not paint or caulk moldy surfaces. Clean up the mold and dry the surfaces before painting. Paint applied over moldy surfaces is likely to peel.

What to Wear When Cleaning Moldy Areas

- Avoid breathing in mold or mold spores. In order to limit your exposure to airborne mold, you may want to wear an N--‐95 respirator, available at many hardware stores and from companies that advertise on the Internet. (They cost about $12 to $25.) Some N--‐95 respirators resemble a paper dust mask with a nozzle on the front, others are made primarily of plastic or rubber and have removable cartridges that trap most of the mold spores from entering. In order to be effective, the respirator or mask must fit properly, so carefully follow the instructions supplied with the respirator. Please note that the Occupational Safety and Health   Administration (OSHA) requires that respirators fit properly (fit testing) when used in an occupational setting; consult OSHA for more information (800--‐321--‐OSHA orwww.osha.gov).

- Wear gloves. Long gloves that extend to the middle of the forearm are recommended. When working with water and a mild detergent, ordinary household rubber gloves may be used. If you are using a disinfectant, a biocide such as chlorine bleach, or a strong cleaning solution, you should select gloves made from natural rubber, neoprene, nitrile, polyurethane, or PVC (see Cleanup and Biocides). Avoid touching mold or moldy items with your bare hands.

- Wear goggles. Goggles that do not have ventilation holes are recommended. Avoid getting mold or mold spores in your eyes.

Mold Remediation Procedure Purpose:

To kill mold caused by flooding, ensure no new mold grows, increase quality of life and health for the homeowner, and general cleaning

Timeline: 3 Days

Supplies:

| Perm. Markers | Goggles | Gloves (work & vinyl) | Paint Trays/Rollers | Brooms/Dust Pans |
| --- | --- | --- | --- | --- |
| Respirators | Filters | Trash Bags | Wire Cutters | Duct Tape |
| Shockwave | Wire Brushes | Shop Towels | Kilz | Ladders (4’, 8’, 10’) |
| Staplers | 5 gal Bucket | Paint Trays | Demo Tools | Hammers |
| Utility Blades | Clorox Wipes |  |  |  |

Steps:

- Remove nails and staples from the face of every top plate, base plate, stud, and ceiling/floor joists. Drive any protruding nails into wooden sub--‐floor.

- All exposed surfaces of the framing need to be scrubbed with a wire brush. Brush up and down, side to side, circularly and diagonally.

  - Once surface is scrubbed, mark an “X” on the stud, indicating a fully scrubbed surface.

  - b. While scrubbing, have some volunteers fold shop towels into sixths.

- Treat the scrubbed wood with Shockwave. Shockwave will come in a soda bottle spray painted green. Mix one bottle of Shockwave with one 5gal bucket of water.

  - Dip a batch of shop towels in the shockwave solution and store them in a paint tray. Wipe down all the wood marked with an “X”. Once a face of the shop towel is dirty, flip it to a different face. Do not double dip used towels into Shockwave solution.

  - When the stud is wiped down on all sides, circle the “X”, and allow the material to dry overnight. 4. Once everything is treated with the Shockwave and allowed time to dry, paint every surface with a thick coat of the Kilz primer.

- Fog the house.

  - Seal the house.

    - Fill all the gaps in the walls with Great Stuff.

    - Use staples and duct tape to secure clear plastic drop cloth over windows and any openings that allow air passage to the exterior of the house (such as soffit vents).

  - Mix Shockwave solution. Add one bottle of Shockwave with one 5gal bucket of water.

  - Put on Tyvek suit, booties, gloves, and mask. Use duct tape or Tyvek tape to tape your booties and gloves to the Tyvek suit.

  - Go inside, close the door, and spray every surface with the solution.

How Do I Know When the Remediation or Cleanup is Finished?

You must have completely fixed the water or moisture problem before the cleanup or remediation can be considered finished.

- You should have completed mold removal. Visible mold and moldy odors should not be present. Please note that mold may cause staining and cosmetic damage.

- You should have revisited the site(s) shortly after cleanup and it should show no signs of water damage or mold growth.

- People should have been able to occupy or re-occupy the area without health complaints or physical symptoms.

Is sampling for mold needed?

In most cases, if visible mold growth is present, sampling is unnecessary. Since no EPA or other federal limits have been set for mold or mold spores, sampling cannot be used to check a building's compliance with federal mold standards. Surface sampling may be useful to determine if an area has been adequately cleaned or remediated. Sampling for mold should be conducted by professionals who have specific experience in designing mold sampling protocols, sampling methods, and interpreting results. Sample analysis should follow analytical methods recommended by the American Industrial Hygiene Association (AIHA), the American Conference of Governmental Industrial Hygienists (ACGIH), or other professional organizations.